Wi-Fi Training Program
Module 3 – Assignment

1. What are the different 802.11 PHY layer standards? Compare their characteristics.

The main 802.11 PHY standards include 802.11a, b, g, n, ac, ax and now 802.11be.

- 802.11a works at 5 GHz with 54 Mbps speed using OFDM.

- 802.11b uses 2.4 GHz with 11 Mbps using DSSS.

- 802.11g also uses 2.4 GHz but offers 54 Mbps using OFDM.

- 802.11n works on both 2.4 GHz and 5 GHz, supports MIMO, and provides up to 600 Mbps.

- 802.11ac uses only 5 GHz, supports wider channels and MU-MIMO, with speeds over 1 Gbps.

- 802.11ax (Wi-Fi 6) uses OFDMA and works on 2.4 and 5 GHz, increasing capacity and efficiency in crowded environments.

- 802.11be (Wi-Fi 7) introduces extremely high throughput, supports 6 GHz band, and advanced modulation.

2. What are DSSS and FHSS? How do they work?

- DSSS (Direct Sequence Spread Spectrum) spreads the signal over a wider bandwidth using a pseudo-random code, making it resistant to interference.

- FHSS (Frequency Hopping Spread Spectrum) rapidly switches the carrier frequency across different channels in a pattern, reducing the chance of interference. Both are early methods for secure and stable wireless transmission.

3. How do modulation schemes work in the PHY layer? Compare different schemes and their performance.

Modulation schemes convert digital data into radio signals. In Wi-Fi, BPSK, QPSK, 16-QAM, 64-QAM, 256-QAM, and now 1024-QAM are used.

BPSK is basic but reliable in low signal conditions.

QPSK is better than BPSK with more data bits per symbol.

16/64/256-QAM increases speed by encoding more bits but needs better signal quality.

Higher modulation increases throughput but is sensitive to noise and interference.

4. What is the significance of OFDM in WLAN? How does it improve performance?

OFDM (Orthogonal Frequency Division Multiplexing) splits a high-speed signal into many lower-speed sub-signals sent over separate sub-carriers. This reduces interference, improves signal stability, and enables faster data rates, especially in multipath environments like buildings.

5. How are frequency bands divided for Wi-Fi? Explain different bands and their channels.

Wi-Fi operates in 2.4 GHz, 5 GHz, and 6 GHz bands.

2.4 GHz has 14 channels (mostly 1, 6, 11 used) with more range but more interference.

5 GHz has more non-overlapping channels (e.g., 36, 40, 44, etc.) and supports faster data.

6 GHz (Wi-Fi 6E) is newly added and provides even more bandwidth with less congestion.

6. What is the role of Guard Intervals in WLAN transmission? How does a short Guard Interval improve efficiency?

Guard Intervals are small gaps between symbol transmissions that prevent interference from reflections (multipath). A shorter Guard Interval (e.g., 400ns) allows faster data transmission with a small risk of overlap, increasing throughput by around 10% if the environment is suitable.

7. Describe the structure of an 802.11 PHY layer frame. What are its key components?

The PHY frame includes PLCP preamble, PLCP header, and the PSDU (Payload).

The preamble helps the receiver synchronize with the signal.

The header includes data rate, length, and other information.

The payload (PSDU) is the actual data to be delivered.

8. What is the difference between OFDM and OFDMA?

- OFDM uses multiple sub-carriers to transmit data for one user at a time.

- OFDMA (Orthogonal Frequency Division Multiple Access), used in Wi-Fi 6, divides those sub-carriers among multiple users simultaneously, increasing efficiency in networks with many devices.

9. What is the difference between MIMO and MU-MIMO?

MIMO (Multiple Input Multiple Output) uses multiple antennas to send and receive more data to/from one device, increasing speed.

MU-MIMO (Multi-User MIMO) goes further by sending data to multiple devices at once, improving performance in multi-user environments.

10. What are PPDU, PLCP, and PMD in the PHY layer?

PPDU (Physical Protocol Data Unit) is the complete unit sent over the air in Wi-Fi.

PLCP (Physical Layer Convergence Protocol) adds headers and preambles to help the MAC layer data adapt to physical transmission.

PMD (Physical Medium Dependent) handles the actual transmission over the wireless channel (modulation, frequency, etc.).

11. What are the types of PPDU? Explain the PPDU frame format across different Wi-Fi generations.

Types of PPDU include Legacy PPDU (802.11a/b/g), HT PPDU (802.11n), VHT PPDU (802.11ac), and HE PPDU (802.11ax).

Each type includes:

Preamble: for synchronization

Header: contains rate, length, and other info

Payload: actual user data

For example, HE PPDU (Wi-Fi 6) includes new fields for OFDMA and MU-MIMO operations, improving efficiency for modern devices.

12. How is the data rate calculated?

Data rate depends on multiple factors like modulation scheme, coding rate, channel width, number of spatial streams, and guard interval.

For example:

Data Rate = (Number of subcarriers) × (Bits per subcarrier) × (Coding rate) × (Symbol rate)

Higher modulation and more spatial streams lead to faster data rates.