Wi-Fi Training Program

Module 4 -  Assignment

1. What is the significance of the MAC layer and in which position is it placed in the OSI model?

The MAC (Medium Access Control) layer is a sublayer of the Data Link Layer (Layer 2) in the OSI model. It is responsible for managing access to the physical transmission medium, handling frame encapsulation/decapsulation, addressing, and ensuring reliable communication over shared media. Its significance lies in coordinating how multiple devices access the medium without collisions and maintaining data integrity at the link level.

2. Describe the frame format of the 802.11 MAC header and explain the purpose of each field.

The 802.11 MAC header consists of several fields:

Frame Control (2 bytes): Indicates the type of frame (management, control, or data), along with other control information such as protocol version, power management, and retry bits.

Duration/ID (2 bytes): Specifies the time for which the medium is reserved or contains association ID.

Address 1 (6 bytes): Usually the destination MAC address.

Address 2 (6 bytes): Usually the source MAC address.

Address 3 (6 bytes): Can be BSSID or another address depending on the frame type.

Sequence Control (2 bytes): Contains sequence number and fragment number for tracking and reassembly.

Address 4 (6 bytes): Used in Wireless Distribution Systems (WDS); present only when four addresses are needed.

QoS Control (2 bytes, optional): Used in QoS-enabled frames for priority control.

HT Control (4 bytes, optional): Used in High Throughput (802.11n+) frames for features like beamforming.

3. Please list all the MAC layer functionalities in the Management, Control, and Data planes.

Management Plane:

Beacon generation

Authentication and Deauthentication

Association and Disassociation

Probing and Scanning

Control Plane:

RTS/CTS (Request to Send / Clear to Send)

Acknowledgment (ACK)

Power Save Poll (PS-Poll)

Control frames for medium access

Data Plane:

Frame aggregation

Fragmentation and reassembly

Error checking (FCS)

Sequence control and retransmissions

4. Explain the scanning process and its types in detail.

Scanning is the process by which a client discovers available wireless networks. There are two types:

Passive Scanning: The client listens for beacon frames periodically transmitted by APs. No frames are sent by the client, conserving battery but possibly taking longer.

Active Scanning: The client sends probe request frames across all channels. APs respond with probe responses. This method is faster and more proactive but consumes more power.

5. Brief about the client association process.

The client association process includes these steps:

Scanning (active or passive) to detect available networks.

Authentication: The client sends an authentication request, and the AP responds with a success or failure.

Association: Once authenticated, the client sends an association request. If accepted, the AP responds with an association response, establishing a connection.

6. Explain each step involved in the EAPOL 4-way handshake and the purpose of each key derived from the process.

The EAPOL 4-way handshake is used in WPA/WPA2 security:

Message 1: The AP sends the ANonce (Authenticator nonce) to the client.

Message 2: The client generates the PTK (Pairwise Transient Key) using ANonce, SNonce (its own nonce), and both MAC addresses, then sends the SNonce to the AP.

Message 3: The AP generates the PTK, verifies it matches the client’s, and sends Group Temporal Key (GTK) encrypted with the PTK.

Message 4: The client confirms receipt and installs the keys.

PMK (Pairwise Master Key): Derived from pre-shared key or EAP process.

PTK: Used for encrypting unicast traffic.

GTK: Used for encrypting broadcast and multicast traffic.

7. Describe the power saving scheme in the MAC layer and explore the types of power saving mechanisms.

The MAC layer supports power saving by allowing clients to enter sleep mode and wake periodically:

Legacy Power Save Mode: Clients sleep and wake at DTIM intervals to receive buffered traffic.

U-APSD (Unscheduled Automatic Power Save Delivery): Used in QoS environments; traffic is delivered when the client sends a trigger frame.

TWT (Target Wake Time): In newer standards (802.11ax), clients negotiate specific wake times, reducing energy consumption significantly.

8. Describe the Medium Access Control methodologies.

Common MAC methodologies in wireless networks include:

CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance): Devices sense the channel before transmitting. If busy, they back off using a random timer.

RTS/CTS: Used to avoid collisions by reserving the medium in advance, especially useful in hidden node scenarios.

DCF (Distributed Coordination Function): A contention-based access mechanism.

PCF (Point Coordination Function): A centralized, polling-based access mechanism used for time-sensitive traffic (rarely used).

9. Brief about the Block ACK mechanism and its advantages.

Block ACK is a feature where multiple frames are acknowledged in a single Block Acknowledgment frame instead of sending an individual ACK for each frame.

Advantages:

- Reduces overhead.

- Increases throughput.

- Minimizes delay in high-data-rate environments.

10. Explain about A-MSDU, A-MPDU, and A-MSDU in A-MPDU.

A-MSDU (Aggregated MAC Service Data Unit): Multiple MSDUs are combined into one MPDU with a single MAC header. Efficient for similar destinations, but if one part is lost, the entire frame is retransmitted.

A-MPDU (Aggregated MAC Protocol Data Unit): Multiple MPDUs are sent in one transmission, each with its own header and checksum, allowing selective retransmission.

A-MSDU in A-MPDU: Combines the benefits of both methods by placing A-MSDUs as payloads within A-MPDUs, improving efficiency and reducing overhead further while allowing error recovery per MPDU.